The University of Azad Jammu & Kashmir, Muzaffarabad

Computer Architecture & Logic Design

| Student Name: | Shahzad Ahmed Awan |
| --- | --- |
| Roll No: | 2024-SE-15 |
| Course Title: | Computer Architecture & Logic Design |
| Course Code: | CS-1205 |
| Instructor: | Engr. Sidra Rafique |
| Submission Date: | 03-June -2025 |
| Department of Software Engineering |  |

Lab 05: HALF ADDER IMPLEMENTATION

## Practical Implementation on Trainer (Using ICs)

- Place the ICs on the breadboard:

  - IC 7486 (XOR gate)

  - IC 7408 (AND gate)

- Provide power connections:

  - Connect Vcc (pin 14) and GND (pin 7) of both ICs.

- Connect input lines:

  - Use two push-button switches or toggle switches for Input A and Input B.

  - Connect these inputs to corresponding pins of both ICs.

- Connect output lines:

  - XOR output pin → LED for Sum

  - AND output pin → LED for Carry

- Use appropriate resistors (typically 330Ω–1kΩ) in series with LEDs to prevent overcurrent.

- Power the board and toggle switches to test combinations:

  - 0 + 0 → Sum = 0, Carry = 0

  - 0 + 1 → Sum = 1, Carry = 0

  - 1 + 0 → Sum = 1, Carry = 0

  - 1 + 1 → Sum = 0, Carry = 1

### 2. Implementation in Electronics Workbench (EWB)

## Part A: Half Adder using Logic Gates

#### Components:

- 2x SPST Switches

- 1x XOR Gate

- 1x AND Gate

- 2x LEDs

- Resistors (220Ω)

- Power Source (+5V)

#### Steps:

- Launch EWB and start a new project.

- Drag and place two SPST switches labeled A and B.

- Insert one XOR gate for Sum output.

- Insert one AND gate for Carry output.

- Connect:

Figure 1: Gate Circuit of HALF ADDER

Inputs A and B to both XOR and AND gates.

  - Output of XOR to LED1 (Sum)

  - Output of AND to LED2 (Carry)

- Connect a 220Ω resistor in series with each LED.

- Attach LEDs to ground to complete the path.

- Power the circuit with a +5V source.

Figure 2: Complicated Circuit Diagram of Half Adder

Test all 4 input combinations (00, 01, 10, 11) using switches and verify the output with the truth table.

## Part B: Half Adder using Digital ICs

#### ICs Used:

- IC 7486 → XOR Gate (for Sum)

- IC 7408 → AND Gate (for Carry)

#### Components:

- IC 7486 (Quad XOR gate)

- IC 7408 (Quad AND gate)

- 2x SPST Switches

- 2x LEDs

- Resistors (220Ω)

- Breadboard (or simulated in EWB)

- Power Supply (+5V and GND)

#### Steps:

- Place IC 7486 and IC 7408 in your circuit.

- Connect Pin 14 of both ICs to +5V, and Pin 7 to GND.

- Use:

  - IC 7486 XOR gate 1 (pins 1, 2 → input; pin 3 → Sum output)

  - IC 7408 AND gate 1 (pins 1, 2 → input; pin 3 → Carry output)

- Inputs A and B are connected to both XOR and AND inputs using SPST switches.

- Outputs (Pin 3 from each IC) go to LEDs with 220Ω resistors.

Figure 3: IC Circuit Implementation of HALF ADDER

Complete the LED connections to ground.

- Test the inputs by toggling switches and compare output LEDs with the truth table.

| A | B | Sum (A⊕B) | Carry (A·B) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

Truth Table of Half Adder